WHERE DATA BECOMES CONSCIOUS

PROJECT 2

Camera-Based

Collision Prevention System

LionChief Dual BLE Train Control — Single Track

Prepared for:  Peter (Piotr) — USA

Prepared by:  Ahmed Ali — CEO, Datix AI Pakistan

Document:  Project 2 — Camera System Documentation

Version:  v10.0 — July 2026

Repository:  github.com/datixai/lionchief-controller-AI-ESP32

DATIX AI  |  Where Data Becomes Conscious  |  datixai.com

# 1.  Project Overview

Project 2 is a camera-based automatic collision prevention system for Peter's model train layout. Two LionChief Bluetooth trains run on the same outer loop track. A USB ceiling camera watches both trains. Python software detects each train's position in real time and automatically controls Train B (ORANGE) speed to maintain a safe gap behind Train A (BLUE).

What the System Does

• Detects both trains using MOG2 motion-based computer vision

• Tracks each train continuously even through turns and speed changes

• Automatically slows ORANGE when it gets too close to BLUE

• Automatically speeds up ORANGE when the gap is too large

• Detects when trains swap positions and handles with ESCAPE mode

• Stops ORANGE in DANGER zone; BLUE keeps running to open the gap

• Auto-pauses when tracker is lost; resumes when both re-acquired

System Components

| Track | Single outer loop — both trains share the same track |
| --- | --- |
| Blue Train (A) | Front leader — BLE controlled, user sets speed |
| Orange Train (B) | Rear follower — gap detection controls speed |
| Camera | USB ceiling camera (index 1) — overhead view of layout |
| Computer | Peter's Windows PC running Python scripts |
| Software | Python + OpenCV + Bleak (BLE library) |
| Folder | single_trackDualBT\ |

BLE Train MAC Addresses

| BLUE Train (A) | 60:FA:11:E3:94:C9 Name: LC-1-1-09F8-60FA |
| --- | --- |
| ORANGE Train (B) | 60:F9:FB:49:94:C9 Name: LC-1-1-09F9-60F9 |

# 2.  PC Setup and File Paths

Peter's PC — Folder Structure

C:\Users\peter\Documents\Linchief_train\

└── single_trackDualBT\

├── main.py              <- main program

├── train_detector.py   <- tracking system

├── ble_controller.py   <- BLE connection

├── speed_controller.py <- gap-to-speed logic

├── config.py           <- all settings

├── calibrate.py        <- distance calibration

├── orange.py           <- orange train test script

├── table_mask.json     <- saved table boundary

└── track_path.json     <- saved track path (optional)

i  If Peter's new PC username is Post-war, the path becomes C:\Users\Post-war\Documents\Linchief_train\single_trackDualBT

Prerequisites — One-Time Installation

| 1 | Install Git | Download from https://git-scm.com/download/win — keep all defaults — restart PC after install |
| --- | --- | --- |
| 2 | Install Python | Download Python 3.11 from https://python.org — check "Add Python to PATH" during install |
| 3 | Clone Repo | cd C:\Users\peter\Documents then git clone https://github.com/datixai/lionchief-controller-AI-ESP32.git Linchief_train |
| 4 | Create venv | cd Linchief_train\single_trackDualBT then python -m venv .venv |
| 5 | Activate venv | Set-ExecutionPolicy -Scope Process -ExecutionPolicy RemoteSigned then .venv\Scripts\activate |
| 6 | Install packages | pip install opencv-contrib-python bleak numpy |

Running Every Session

cd C:\Users\peter\Documents\Linchief_train

.venv\Scripts\activate

cd single_trackDualBT

python main.py

i  After git pull, no reinstallation needed. Just activate venv and run.

# 3.  Running the Program

Every Session — Start Commands

| 1 | BLE Connects | Both trains found and connected. Terminal shows "[OK] TrainA Connected" and "[OK] TrainB Connected" |
| --- | --- | --- |
| 2 | Camera Warms Up | 40 frames captured silently so auto-exposure settles — prevents camera flicker |
| 3 | Table Mask Loads | Saved table boundary loaded automatically. Terminal shows "Table mask loaded" |
| 4 | Window Opens | Camera view appears. Screen shows STEP 2 — drag box on BLUE Train |
| 5 | Draw Boxes | User drags boxes on both trains (see Section 4) |
| 6 | Y / N Prompt | After both trains selected — system asks to start |
| 7 | Running | Auto mode controls ORANGE train speed based on gap to BLUE |

Startup Checklist

• Both trains powered ON and on the track

• Phone Bluetooth is OFF — it can steal the BLE connection

• Camera plugged in and ceiling-mounted with full track view

• Trains separated by a reasonable distance (not touching)

• Table lamp is ON — camera needs good lighting

# 4.  Drawing Boxes — Step by Step

Step 1 — Table Boundary (First Run Only)

• Press T key to enter table drawing mode

• Hold left mouse button and drag a rectangle around the track oval

• Release — saved automatically to table_mask.json

• Next runs: loads automatically

i  Draw the boundary TIGHT around the track oval only — not the whole table. This keeps people standing at the edge outside the detection zone.

Step 2 — Draw Box on BLUE Train (Front Leader)

• Screen shows:  STEP 2 -- Drag a box around BLUE Train

• Find the BLUE train on camera

• Hold left mouse button, drag a box around the BLUE train body, release

• Screen flashes:  BLUE Train locked — now select ORANGE Train

• White tracking circle appears on BLUE train

Step 3 — Draw Box on ORANGE Train (Rear Follower)

• Screen shows:  STEP 3 -- Drag a box around ORANGE Train

• Hold left mouse button, drag a box around the ORANGE train body, release

• Screen flashes:  ORANGE Train locked — both tracking!

• Orange tracking circle appears on ORANGE train

Step 4 — Y / N Confirmation

| Y | AUTO mode — gap detection controls ORANGE train speed automatically |
| --- | --- |
| N | MANUAL mode — you control both train speeds yourself |

If Box Goes Wrong

• Press A key → drag new box on BLUE train

• Press B key → drag new box on ORANGE train

• Press T key → redraw the table boundary

# 5.  Keyboard Controls

BLUE Train — Front Leader

| [ | Decrease BLUE train speed by 1 |
| --- | --- |
| ] | Increase BLUE train speed by 1 |
| F1 | Stop BLUE train immediately |
| F2 | Resume BLUE train at last speed |

ORANGE Train — Rear Follower

| 1 – 7 | Set ORANGE to exact speed (1=slowest 7=full power) |
| --- | --- |
| + or = | Speed up ORANGE by 1 |
| - or _ | Slow down ORANGE by 1 |
| Arrow Up | Speed up ORANGE by 1 |
| Arrow Down | Slow down ORANGE by 1 |
| S | Stop ORANGE train |
| R | Resume ORANGE train at last speed |

Shared Controls

| M | Toggle AUTO mode / MANUAL mode |
| --- | --- |
| E | Emergency stop — BOTH trains immediately |
| A | Re-select BLUE train (drag new box) |
| B | Re-select ORANGE train (drag new box) |
| T | Redraw table boundary |
| K | Toggle track path recording |
| H | Horn — both trains |
| L | Lights — both trains |
| Q or ESC | Quit program |

# 6.  Speed Zones — How Auto Control Works

The system measures the pixel gap between the two train tracking circles (surface to surface). Based on this gap it assigns one of five zones and controls ORANGE train speed accordingly.

| Zone | Gap | Orange Speed | What Happens |
| --- | --- | --- | --- |
| DANGER | < 100 px | STOP | ORANGE stops. BLUE keeps running to open the gap. |
| WARNING | 100-160 px | Speed 2 | ORANGE slows to minimum speed. |
| CAUTION | 160-230 px | Speed 3 | ORANGE reduces speed. |
| SAFE | 230-320 px | User speed | ORANGE follows at user-set speed. |
| FAR | > 320 px | Speed 7 | Gap too large — ORANGE catches up at full power. |

PD Controller and BLE Latency

The system also measures how fast the gap is closing per frame. If trains approach quickly it brakes earlier. A 150ms BLE latency compensation is built in — commands are sent slightly early to account for response delay.

# 7.  Configuration Settings (config.py)

Train BLE Settings

| TRAIN_A_MAC | 60:FA:11:E3:94:C9 (BLUE train) |
| --- | --- |
| TRAIN_A_NAME | LC-1-1-09F8-60FA |
| TRAIN_B_MAC | 60:F9:FB:49:94:C9 (ORANGE train) |
| TRAIN_B_NAME | LC-1-1-09F9-60F9 |

Camera Settings

| CAMERA_INDEX | 1 — change to 0 if wrong camera opens |
| --- | --- |
| DISPLAY_W / H | 960 x 540 — never change these |
| CAMERA_WARMUP_FRAMES | 40 — frames read silently at startup |

Tracking Sensitivity

| SEARCH_RADIUS | 120 px — increase if fast train loses tracking |
| --- | --- |
| MIN_BLOB_AREA | 60 px — decrease if trains not detected |
| MOG2_VAR_THRESHOLD | 50 — increase to ignore more background noise |

Speed Settings

| DEFAULT_SPEED_A / B | 5 — starting speed for each train |
| --- | --- |
| Speed 1 byte | 0x04 (~14% of max power) |
| Speed 7 byte | 0x1C (100% full power) |
| BLE_LATENCY_MS | 150ms — time between command sent and train responds |

# 8.  Known Problems and Limitations

1. Tracking Loses Train on Sharp Turns

MOG2 detects motion blobs. On tight curves the train may move faster than the search radius can follow.

• Workaround: increase SEARCH_RADIUS in config.py to 150

• Workaround: press A or B to re-draw the box on the lost train

2. Both Trains Touching — Blob Merges

When trains are physically touching, MOG2 sees one large blob. The system cannot reliably separate them.

!  This is a fundamental limitation of motion-based tracking. The only 100% reliable solution is ArUco markers — printed squares on each train roof giving the camera unique IDs.

3. People Moving Near Table

MOG2 detects all motion inside the table boundary. If a person reaches across, the motion blob can pull a tracker off the train.

• Fix: draw table boundary TIGHT around the track oval only

4. Train Stops on Hill

Gap control may reduce ORANGE speed exactly when it needs more power for an uphill section.

• Fix: manually increase speed with ] key before the hill section

# 9.  Orange Train Standalone Test (orange.py)

A separate test file is included to test the ORANGE train on its own without collision prevention.

cd C:\Users\peter\Documents\Linchief_train\single_trackDualBT

.venv\Scripts\activate

python orange.py

i  If ORANGE connects and responds in orange.py but not in main.py, the problem is in the dual BT logic — not the train itself.

# 10.  Troubleshooting

| Problem | Solution |
| --- | --- |
| Train not connecting | Power the train ON. Turn phone Bluetooth OFF. Wait 15 seconds. |
| python is not recognized | Python not installed or not added to PATH. Reinstall Python, check "Add Python to PATH". |
| git is not recognized | Git not installed. Download from git-scm.com and restart PC. |
| Camera opens wrong device | Change CAMERA_INDEX in config.py from 1 to 0 (or 2). |
| Tracking boxes wrong position | Window must be at 960x540. Delete table_mask.json and redraw. |
| Both trains stop and will not restart | Press E to emergency stop. Re-draw boxes. Trains were too close. |
| DANGER fires constantly | Move trains further apart before drawing boxes. Grace period is 5 seconds. |
| Train B not responding | Run orange.py to test ORANGE train alone. |
| git pull says everything up to date | Ahmed has not pushed new code yet. Contact Datix AI. |
| Camera flickers on open | Normal — 40 warm-up frames. Wait 2-3 seconds. |

Document prepared by Datix AI  |  Ahmed Ali  |  July 2026

For support contact Ahmed Ali — Datix AI Pakistan