WHERE DATA BECOMES CONSCIOUS

PROJECT 3

Web Application for

LionChief Dual BLE

Train Control

Browser-Based Monitoring and Control Dashboard

Prepared for:  Peter (Piotr) — USA

Prepared by:  Ahmed Ali — CEO, Datix AI Pakistan

Document:  Project 3 — Web Interface Documentation

Version:  v1.0 — July 2026

Repository:  github.com/datixai/lionchief-controller-AI-ESP32

DATIX AI  |  Where Data Becomes Conscious  |  datixai.com

# 1.  Project Overview

Project 3 is a web-based control dashboard for the LionChief dual BLE train system. Instead of running a Python terminal window and using keyboard shortcuts, Peter opens a browser page that shows the live camera feed and all controls as clickable buttons — like a professional monitoring dashboard.

What Changed from Project 2

| Project 2 | python main.py → OpenCV window → keyboard shortcuts |
| --- | --- |
| Project 3 | python web_app.py → browser at localhost:5000 → buttons |
| Same underneath | All tracking, BLE, and gap detection logic is identical |

What the Dashboard Shows

• Live camera feed from the ceiling USB camera

• Colour-coded zone indicator — DANGER / WARNING / CAUTION / SAFE / FAR

• Real-time gap distance in pixels between the two trains

• Speed bars and confidence indicators for each train

• BLE connection status — green dot when connected

• Step-by-step instructions overlaid on the camera

• All speed and control buttons below the camera — no scrolling

Project Folder — File List

| web_app.py | Main Flask server — replaces main.py |
| --- | --- |
| index.html | Browser dashboard — all CSS and JS included inside |
| config.py | All settings — MACs, distances, speeds |
| train_detector.py | MOG2 camera tracking system (same as Project 2) |
| speed_controller.py | Gap-to-speed logic with PD controller (same as Project 2) |
| ble_controller.py | Dual BLE train connection (same as Project 2) |
| start.bat | Double-click launcher — opens browser automatically |

i  The templates\ and static\ subfolders are NOT needed. index.html sits in the root lionchefapp\ folder alongside all .py files.

# 2.  How to Run — Every Session

Method 1 — Double-Click start.bat (Recommended)

This is the easiest way. No terminal knowledge needed.

| 1 | Find the file | Open C:\Users\peter\Documents\Linchief_train\lionchefapp\ in Windows Explorer |
| --- | --- | --- |
| 2 | Double-click | Double-click start.bat |
| 3 | Allow it | If Windows shows a warning, click "More info" then "Run anyway" |
| 4 | Wait | A black terminal window opens. Wait 3-4 seconds. |
| 5 | Browser opens | Browser opens automatically at http://localhost:5000 |
| 6 | Done | Camera loads, dashboard is ready to use |

i  The virtual environment (.venv) is already set up in the root Linchief_train\ folder. start.bat finds it automatically — no setup needed each time.

Method 2 — VS Code Terminal

Use this if start.bat does not work for any reason.

cd C:\Users\peter\Documents\Linchief_train

.venv\Scripts\activate

cd lionchefapp

python web_app.py

Then open browser manually and go to:

http://localhost:5000

How to Stop the App

• Press Ctrl + C in the terminal window

• Both trains disconnect safely

• Close the browser tab

! Do not just close the terminal window without pressing Ctrl+C — trains may stay at their last speed.

Getting Updates from GitHub

When Datix AI pushes new code, Peter runs:

cd C:\Users\peter\Documents\Linchief_train

git pull

Then double-click start.bat as normal. No reinstallation needed.

# 3.  How the System Works

Architecture Overview

When start.bat runs, two things happen simultaneously in the background:

| Python Backend | Browser Frontend |
| --- | --- |
| • Captures camera frames • Runs MOG2 train tracking • Calculates gap between trains • Sends BLE speed commands • Draws overlay on camera frame • Streams JPEG video to browser | • Displays live camera stream • Shows speed and zone in real time • Receives status via WebSocket • Sends button clicks to Python • Mouse drag selects trains on camera • Works in any modern browser |

Startup Sequence

| 1 | BLE Connects | web_app.py connects to Blue and Orange trains via Bluetooth. Green dots appear in browser header when connected. |
| --- | --- | --- |
| 2 | Camera Warms Up | 40 frames captured silently so auto-exposure settles. Prevents camera flicker. |
| 3 | Table Mask Loads | Saved table boundary loaded from table_mask.json automatically. |
| 4 | Dashboard Ready | Browser shows camera feed. Step 2 instruction appears — drag box on Blue train. |
| 5 | User Selects Trains | User drags boxes on camera image in browser. Coordinates sent to Python. |
| 6 | Y / N Prompt | After both trains selected, Start AUTO or Start MANUAL popup appears. |
| 7 | Running | Auto mode controls Orange train speed. Blue train runs at user-set speed. |

# 4.  Selecting Trains on the Dashboard

Step 1 — Draw Table Boundary (First Run Only)

• Press T button on the dashboard

• Hold left mouse button and drag a rectangle around the track oval

• Release — boundary saved to table_mask.json automatically

• Next sessions: loads automatically, no need to redraw unless camera is moved

i  Draw the boundary tight around the track only — not the whole table. This prevents people walking nearby from disturbing the tracker.

Step 2 — Select Blue Train (Front Leader)

• Camera bar shows: Step 2 — Drag a box around BLUE Train

• Find the Blue train in the camera image

• Hold left mouse button and drag a rectangle around the Blue train

• Release — white tracking circle appears on Blue train

• Dashboard flashes: BLUE Train locked — now select ORANGE Train

Step 3 — Select Orange Train (Rear Follower)

• Camera bar shows: Step 3 — Drag a box around ORANGE Train

• Drag a rectangle around the Orange train

• Release — orange tracking circle appears on the train

• Dashboard flashes: ORANGE Train locked — both tracking!

Step 4 — Start Confirmation

| START AUTO | Gap detection controls Orange train speed automatically. Recommended for normal operation. |
| --- | --- |
| START MANUAL | You control both train speeds manually using the buttons and keyboard. |

If a Box Goes Wrong

• Click Reselect Blue → drag new box on Blue train

• Click Reselect Orange → drag new box on Orange train

• Click Table → redraw the table boundary

# 5.  Dashboard Controls

Blue Train Buttons — Left Panel

| Slower | Decrease Blue train speed by 1 |
| --- | --- |
| Faster | Increase Blue train speed by 1 |
| Stop | Stop Blue train immediately |
| Resume | Resume Blue train at last speed |

Orange Train Buttons — Right Panel

| 1 2 3 4 5 6 7 | Set Orange train to exact speed level |
| --- | --- |
| Slower | Decrease Orange train speed by 1 |
| Faster | Increase Orange train speed by 1 |
| Stop | Stop Orange train |
| Resume | Resume Orange train at last speed |

Shared Buttons — Centre Panel

| AUTO / MANUAL | Toggle between auto gap detection and manual control |
| --- | --- |
| Stop Both | Emergency stop — both trains immediately |
| Reselect Blue | Press then drag new box on Blue train |
| Reselect Orange | Press then drag new box on Orange train |
| Table | Redraw the table boundary mask |
| Horn | Toggle horn on both trains |
| Lights | Toggle lights on both trains |

Keyboard Shortcuts (also work in the browser window)

| [ and ] | Blue train slower / faster |
| --- | --- |
| F1 and F2 | Blue train stop / resume |
| 1 — 7 | Set Orange train speed (1=slowest 7=full power) |
| + and - | Orange train faster / slower |
| S and R | Orange train stop / resume |
| Arrow Up / Down | Orange train faster / slower |
| M | Toggle AUTO / MANUAL mode |
| E | Emergency stop — both trains immediately |
| A | Reselect Blue train |
| B | Reselect Orange train |
| T | Redraw table boundary |
| H | Horn on both trains |
| Y / N | Confirm Start AUTO or Start MANUAL |

# 6.  Speed Zones and Auto Control

In AUTO mode the system measures the pixel gap between the two train circles and controls Orange train speed automatically using a PD controller.

| Zone | Gap | Orange Speed | What Happens |
| --- | --- | --- | --- |
| DANGER | < 100px | STOP | Orange stops. Blue keeps running forward to open the gap. |
| WARNING | 100-160px | Speed 2 | Orange slows to minimum speed. |
| CAUTION | 160-230px | Speed 3 | Orange reduces speed cautiously. |
| SAFE | 230-320px | User speed | Orange follows at the user-set speed. |
| FAR | > 320px | Speed 7 | Gap too large — Orange catches up at full power. |

PD Controller and BLE Latency

The system also measures how fast the gap is closing per frame (derivative term). If trains are approaching quickly it brakes earlier. A 150ms BLE latency compensation is built in — commands are sent slightly early to account for the response delay.

# 7.  Troubleshooting

| Problem | Solution |
| --- | --- |
| start.bat blocked by Windows | Right-click start.bat → Properties → check Unblock → OK |
| Virtual environment not found | Run from VS Code terminal: cd Linchief_train → .venv\Scripts\activate → cd lionchefapp → python web_app.py |
| Browser does not open | Type http://localhost:5000 manually in Chrome or Edge |
| Camera opens wrong device | Change CAMERA_INDEX in config.py from 1 to 0 or 2 |
| Train not connecting (grey dot) | Power the train ON, turn phone Bluetooth OFF, wait 15 seconds |
| Tracking circle jumps off train | Click Reselect Blue or Reselect Orange and drag a new box |
| Both trains stop and will not restart | Press E (emergency stop), wait 5 seconds, redraw boxes with trains further apart |
| git pull says everything up to date | Ahmed has not pushed yet — contact Datix AI for confirmation |
| Page looks broken | Use Chrome or Edge — Internet Explorer is not supported |

Document prepared by Datix AI  |  Ahmed Ali  |  July 2026

For support contact Ahmed Ali — Datix AI Pakistan