WHERE DATA BECOMES CONSCIOUS

Note

DRAFT — NOT FINALISED PROPOSAL

This document is for discussion purposes only. Design is subject to change.

PROJECT 4

Large Scale Attic Layout

Collision Prevention System

Sensor-Based vs Camera-Based Approach Analysis — 100m Track / 3 Trains

Prepared for:  Peter (Piotr) — USA

Prepared by:  Ahmed Ali:  CEO, Datix AI Pakistan

Document Type:  Preliminary Technical Proposal:  For Discussion Only

Date:  August 2026

Status:  DRAFT , Not Finalised

DATIX AI  |  Where Data Becomes Conscious  |  datixai.com

IMPORTANT NOTICE — DRAFT DOCUMENT

This document is a PRELIMINARY PROPOSAL only. It is intended to outline possible technical approaches, answer Harry's initial questions, and provide a basis for further discussion. No designs, specifications, or cost estimates in this document are final. All details are subject to change following:

•  Site visit and physical inspection of the attic space

•  Confirmation of the final track plan and dimensions

•  Discussion of budget and preferred technology direction

•  Testing and evaluation of approach on a small section first

!  Do not purchase any hardware based solely on this document. Hardware recommendations are indicative only. A finalised specification will be provided before any purchase is made.

# 1.  Executive Summary

Harry is planning a significantly larger model train layout — approximately 100 metres of total track in an attic space, with a main loop of around 20 metres. The goal is to run 2 to 3 trains simultaneously on this layout without collisions. This document evaluates two distinct technical approaches and answers each of Harry's specific questions.

Layout at a Glance

| Location | Attic (new layout) |
| --- | --- |
| Main line length | ~20 metres |
| Total track length | ~100 metres |
| Track complexity | High — complex loops, multiple sidings |
| Simultaneous trains | 2 to 3 maximum |
| Train type | LionChief — Bluetooth (BLE) |
| Target completion | August 2026 (track laying) |

Two Approaches Evaluated

| Option A — Sensor-Based | Option B — Camera-Based |
| --- | --- |
| Reed switches at block boundaries ESP32 microcontroller brain 100% reliable, no light dependency Low cost, proven technology Recommended for this layout | Multiple overhead cameras Dedicated PC for processing Visual tracking of train positions Higher cost, complex setup Feasible but not recommended |

# 2.  Harry's Questions — Answered

Q1:  How many sections and sensors would make sense?

The track should be divided into blocks using a concept called Block Signaling — the same approach used in real railways. Each block needs one sensor at its entry point. A train entering a block triggers the sensor; the system then knows that block is occupied.

Rule of thumb: you need at least 3x the number of trains in blocks so trains always have empty space between them.

| Trains Running | Minimum Blocks | Recommended Blocks |
| --- | --- | --- |
| 2 trains | 6 | 10–12 |
| 3 trains | 9 | 12–15 |

i  For Harry's layout: 12 to 15 blocks recommended. At 100m total track ÷ 12 blocks = ~8m per block average. Each block needs 1 reed switch sensor = 12 to 15 sensors total.

Q2:  Which sensor type is recommended?

Three sensor types are used in model railroads. Reed switches are the clear winner for this application:

| Sensor Type | Cost | Reliability | Notes |
| --- | --- | --- | --- |
| Reed Switch | $0.50–$1 each | 100% — mechanical | Magnet under loco. No track wiring. Works anywhere. |
| IR Sensor | $1–$3 each | Good — optical | Affected by ambient light. Needs alignment under track. |
| Hall Effect | $1–$2 each | Very good | Similar to reed switch. Slightly more complex wiring. |

i  RECOMMENDATION: Reed switches (model MC-38). A small magnet is glued under each locomotive. When the train passes the sensor, the magnet closes the reed switch and the ESP32 detects it instantly.

Q3:  Dedicated PC or ESP32 microcontroller?

For the sensor-based approach, an ESP32 microcontroller is completely sufficient and is in fact the better choice. No PC is required.

| ESP32 Microcontroller | Dedicated PC |
| --- | --- |
| Cost: $5–8 Built-in WiFi for web dashboard 15+ GPIO pins for sensors BLE to control trains directly Runs 24/7 on USB power bank No screen, no keyboard needed | Cost: $300–600 Required ONLY for camera approach Overkill for sensor logic Needs display and peripherals Higher power consumption More complex to maintain |

i  RECOMMENDATION: One ESP32 as the master controller. It reads all reed switches, runs block logic, sends BLE commands to trains, and serves the web dashboard — all in one $8 device.

Q4:  Will Bluetooth (BLE) work reliably at 20m in an attic?

BLE range indoors is typically 10–15 metres. A 20-metre attic with metal beams and interference makes reliable BLE coverage across the full layout uncertain. Three strategies are available:

| Strategy 1 | Place ESP32 in the centre of the layout (~10m to each end). Most cases this will work. |
| --- | --- |
| Strategy 2 | Use two ESP32 units — one at each end — and coordinate them via WiFi. |
| Strategy 3 | If buying new locomotives for this layout, switch to DCC. Signal runs through the track rails — unlimited range, industry standard for large layouts. |

!  DRAFT NOTE: BLE range must be tested physically in the attic before finalising this approach. Strategy will be decided after on-site testing.

# 3.  Option A — Sensor-Based System (Recommended)

This approach divides the track into blocks using reed switch sensors. The ESP32 always knows which blocks are occupied and controls train speeds accordingly. It is the same principle used in real railway signaling.

How It Works — Step by Step Workflow

| 1 | Train moves | A magnet under the locomotive approaches a reed switch at a block boundary |
| --- | --- | --- |
| 2 | Sensor fires | Reed switch closes → GPIO pin on ESP32 goes HIGH → block marked OCCUPIED |
| 3 | Block logic runs | ESP32 checks: is the NEXT block occupied? Is the block after that occupied? |
| 4 | Speed decision | If next block clear → maintain speed. If next block occupied → slow down. If same block → emergency stop. |
| 5 | BLE command | ESP32 sends speed command via Bluetooth directly to the affected LionChief locomotive |
| 6 | Train responds | Locomotive adjusts speed within 150ms of receiving command |
| 7 | Block clears | When train exits a block, the EXIT sensor fires → block marked CLEAR → following train can resume |
| 8 | Dashboard | Web page on any phone shows live block map — green = clear, red = occupied |

Block Map — What the System Tracks

The layout is divided into named blocks. Each block has an entry sensor. The system maintains a live map like this:

| B01 | B02 | B03 | B04 | B05 | B06 | B07 | B08 | B09 | B10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CLEAR | TRAIN 1 | CLEAR | CLEAR | TRAIN 2 | CLEAR | CLEAR | TRAIN 3 | CLEAR | CLEAR |

Red = occupied by a train   |   Green = clear   |   System automatically slows any train approaching a red block.

Hardware Required

| ESP32 dev board | 1 unit, master controller, WiFi server, BLE hub |
| --- | --- |
| Reed switches (MC-38) | 15–18 units, one per block entry point |
| Small round magnets | One per locomotive (glued under body) |
| 22AWG wire | 50–80 metres to wire sensors back to ESP32 |
| USB power supply | 5V 2A, to power ESP32 permanently |
| Junction box | To house ESP32 and wiring neatly |

i  IMPORTANT: Reed switches must be installed DURING track laying in September — much easier than retrofitting after track is fixed. Coordinate sensor placement with track installation.

Advantages of Sensor-Based Approach

•  100% reliable: no cameras, no lighting dependency, no vision algorithms

•  Works in any light level — attic lighting irrelevant

•  Works at any track size — add more sensors as layout grows

•  Very low cost — under $50 total hardware

•  Proven technology — used in real model railroad clubs worldwide

•  No PC required — ESP32 handles everything

Limitations

•  Sensors must be installed during track laying — harder to retrofit

•  Does not know WHERE on the track the train is between sensors

•  Block resolution limited to sensor placement spacing

# 4.  Option B: Camera-Based System

This approach uses overhead cameras to visually track train positions in real time, similar to what was built in Projects 2 and 3, but scaled up significantly for the large attic layout.

!  DRAFT NOTE: The camera approach was the method used in Projects 2 and 3 on the small oval layout. While it worked as a proof of concept, it had reliability challenges even on a small layout. Scaling it to 100m of complex track would require significant investment in hardware and engineering.

How It Would Work, Workflow

| 1 | Cameras capture | 3–5 ceiling cameras positioned above different sections of the layout capture continuous video at 30fps |
| --- | --- | --- |
| 2 | Frame stitching | PC software stitches camera feeds into one unified overhead view of the full layout |
| 3 | Train detection | Computer vision (MOG2 or ArUco markers) locates each train in the unified frame |
| 4 | Gap calculation | Software measures pixel distances between trains in each section of track |
| 5 | Speed decision | Gap-to-speed algorithm (PD controller) determines required speed for each train |
| 6 | BLE command | PC sends Bluetooth speed commands to each locomotive |
| 7 | Dashboard | Web browser shows unified camera view with tracking circles and speed controls |

Challenges at 100m Scale

| Challenge | Why it is difficult |
| --- | --- |
| Multiple cameras needed | One camera cannot cover 100m of complex track. 3–5 cameras needed, each requiring its own USB connection to the PC. |
| Camera stitching | Combining 5 camera feeds into one unified image requires significant software and calibration work. |
| Train identification | At small scale, trains look similar from above. At 100m scale this problem is worse. ArUco markers on each locomotive would help. |
| BLE range | PC would need to maintain BLE connections to 3 trains across a 20m space with potential interference. |
| Processing power | Running 5 camera streams simultaneously with real-time CV processing requires a capable dedicated PC. |
| Lighting consistency | Attic lighting is uneven. Poor lighting causes false detections. Consistent overhead LED lighting would be required. |
| Cost | 3–5 cameras + capable PC + LED lighting = $500–1000+ investment. |

Hardware Required for Camera Approach

| Dedicated PC | High spec laptop or mini PC — Core i5, 16GB RAM minimum |
| --- | --- |
| USB cameras | 3–5x 4K USB cameras (wide angle) |
| USB hub | 7-port powered USB hub for multiple cameras |
| LED lighting | Consistent overhead LED strips for even illumination |
| Camera mounts | Ceiling mounts or tripods above each section |
| Calibration targets | Reference markers on layout for camera alignment |

When Camera Approach Makes Sense

•  Layout has good fixed overhead mounting positions for cameras

•  Budget allows for $500–1000+ hardware investment

•  You want visual monitoring of the layout regardless of collision prevention

•  Track is already laid and sensors cannot be easily installed

# 5.  Side-by-Side Comparison

| Criteria | Option A — Sensors | Option B — Camera | Best |
| --- | --- | --- | --- |
| Hardware cost | $44 approx | $500–1000+ | S |
| Reliability | 100% mechanical | 85–90% , variable | S |
| Light dependency | None | High — needs good lighting | S |
| PC required | No ESP32 only | Yes — dedicated PC | S |
| Scalability | Excellent — add sensors | Hard — add cameras | S |
| Installation | During track laying | After track laid | S |
| Visual monitoring | No — block map only | Yes — see full layout | C |
| Setup complexity | Low | Very high | S |
| Maintenance | Very low | Medium–high | S |
| Attic suitability | Excellent | Requires good lighting | S |
| Future expansion | Easy | Complex | S |
| Proven for 100m | Yes — railway standard | Experimental at this scale | S |

| S | Sensor wins this criterion | C | Camera wins this criterion |
| --- | --- | --- | --- |

# 6.  Recommended Architecture — Sensor System

Based on the analysis above, the sensor-based approach is recommended for the Project 4 attic layout. Below is the proposed system architecture.

System Block Diagram

TRACK LAYER [Reed Switch 1] [Reed Switch 2] ... [Reed Switch 15] Magnet under each locomotive closes switch as train passes

▼ GPIO pins wired back to ESP32 ▼

ESP32 MASTER CONTROLLER ($8) Block Detection Logic ¦ Collision Prevention ¦ WiFi Web Server ¦ BLE Hub to Trains No PC required — Powered by USB — Runs 24/7

▼ Bluetooth BLE to trains ▼ ▼ WiFi to phone ▼

OUTPUTS [ LionChief Trains — Speed commands via BLE ] [ Phone / Tablet — Live block map in browser ] Any phone on same WiFi network — no app install needed

Web Dashboard — What Peter Sees

The ESP32 hosts a simple web page accessible on any phone or tablet connected to the same WiFi network. No app installation needed.

| Live block map | Colour-coded grid: green = clear, red = occupied, shows which block each train is in |
| --- | --- |
| Train speeds | Current speed of each train displayed |
| Manual override | Buttons to increase/decrease any train speed |
| Emergency stop | One button stops all trains immediately |
| BLE status | Shows which trains are connected |
| Access | http://192.168.1.xxx — any phone browser on same WiFi |

# 7.  Indicative Hardware List

!  DRAFT — Do not purchase based on this list alone. Final specification will be provided after track plan is confirmed.

Sensor-Based System — Indicative Cost

| Item | Qty | Unit Cost | Total | Priority |
| --- | --- | --- | --- | --- |
| ESP32 development board (38-pin) | 2 | $5 | $10 | Essential |
| Reed switches MC-38 | 20 | $0.50 | $10 | Essential |
| Small round magnets 6mm | 10 | $0.50 | $5 | Essential |
| 22AWG 2-core wire (50m roll) | 2 | $8 | $16 | Essential |
| 5V 2A USB power supply | 1 | $8 | $8 | Essential |
| Small plastic junction box | 1 | $5 | $5 | Recommended |
| Breadboard + jumper wires | 1 | $5 | $5 | Recommended |
| USB-C cable 2m | 2 | $3 | $6 | Recommended |
| Spare reed switches | 5 | $0.50 | $3 | Spare |
| TOTAL ESTIMATED COST |  |  | ~$68 |  |

Proposed Timeline

| Month | Phase | Key Activities |
| --- | --- | --- |
| August 2026 | Hardware Order | Finalise block plan from track layout. Order ESP32, reed switches, wire. Begin firmware development. |
|  | Track Laying | Install reed switches DURING track laying. Much easier than retrofitting. Coordinate with track installation. |
|  | Integration | Connect sensors to ESP32. Upload firmware. Test block detection on completed track. |
|  | Testing | Multi-train testing. Tune block boundaries. Calibrate BLE range. Launch web dashboard. |
|  | Final | Full system running. Documentation. Handover. |

# 8.  Next Steps — Discussion Required

Before this proposal can be finalised, the following points need to be discussed and confirmed between Harry and Datix AI:

| 1 | Confirm approach: Sensor-based (recommended) or Camera-based? |
| --- | --- |
| 2 | Confirm final track plan — block boundaries need to be mapped against the green track drawing |
| 3 | Confirm number of simultaneous trains — 2 or 3? This determines number of blocks needed |
| 4 | Confirm if LionChief locomotives will be reused or if new locomotives (potentially DCC) will be purchased |
| 5 | Confirm BLE range test — ESP32 position in the attic needs to be evaluated |
| 6 | Confirm budget for Project 4 hardware |
| 7 | Confirm September track laying date — sensor installation must be coordinated with this |

DRAFT — NOT FINALISED — FOR DISCUSSION ONLY

Document prepared by Datix AI  |  Ahmed Ali  |  August 2026

For discussion contact Ahmed Ali — Datix AI Pakistan