|  | DATIX AI ¦ Engineering Division Intelligent Systems • Embedded AI • IoT Automation |
| --- | --- |

Harry Locomotive Project Firmware v5.1 — Changes & Full Pin Reference Prepared by Ahmed Ali, CEO — Datix AI ¦ May 2026

|  | WHAT CHANGED IN v5.1 |
| --- | --- |

| Version | Change | Detail |
| --- | --- | --- |
| v5.1 #1 | Resume at user-set speed | Before any STOP, the current speed is saved. After inner train parks and outer resumes, it returns to exactly the speed you had set — not a fixed slow speed. Set speed 5, get speed 5 back. |
| v5.1 #2 | Inner train auto-runs every 30s | No more pressing X every time. Inner train starts automatically every 30 seconds if the zone is clear and outer is not in shared section. X key still works for immediate manual trigger. |
| v5.1 #3 | Keepalive bug fixed | Keepalive was sending speed 0 when outer train had been stopped, which was silently stopping the train every 20 seconds. Fixed — keepalive only sends if speed > 0. |
| v5.1 #4 | GPIO corrected | RELAY_POWER_PIN changed from GPIO18 to GPIO19 to match Peter's actual wiring. RELAY_SWITCH_PIN set to GPIO18 (track switch, not yet connected). |

|  | COMPLETE PIN REFERENCE — Every Wire Explained |
| --- | --- |

| GPIO Pin | Wire Color | Connected To | What It Does |
| --- | --- | --- | --- |
| GPIO 15 | Black | Sensor B signal | Outer loop entry — only outer train passes this sensor |
| GPIO 16 | Black | Sensor A signal | Inner loop entry — only inner train passes this sensor |
| GPIO 17 | Black | Sensor C signal | Shared section exit — any train passing here signals zone clear |
| GPIO 18 | Any | Relay 2 IN | Track switch 1-second pulse — not yet physically connected |
| GPIO 19 | Any | Relay 1 IN | Inner train track power — cuts power to stop inner train |
| GPIO 2 | — | Built-in LED | Status indicator — blinks = running, solid = BLE connected |
| VIN | Brown | All 3 sensors VCC | 5V power for E18-D80NK sensors (brown wire on each) |
| VIN | Red/Any | Relay 1 + 2 VCC | 5V power for both relay modules |
| GND | Blue | All 3 sensors GND | Common ground for E18-D80NK sensors (blue wire on each) |
| GND | Black | Relay 1 + 2 GND | Common ground for both relay modules |

E18-D80NK Sensor Wire Colours (same on all 3 sensors): Brown wire → VIN (5V) Blue wire → GND Black wire → GPIO signal pin Relay modules: Standard Arduino relay = ACTIVE LOW. HIGH = relay off (safe default). LOW = relay activates.

|  | RELAY WIRING DETAIL |
| --- | --- |

| Relay 1 — GPIO 19 — Inner Train Power Terminal used: NC (Normally Closed) COM → inner loop track power wire 1 NC → inner loop track power wire 2 GPIO19 HIGH → NC closed → power flows → train runs GPIO19 LOW → NC open → power cut → train stops | Relay 2 — GPIO 18 — Track Switch Terminal used: NO (Normally Open) COM → track switch contact 1 NO → track switch contact 2 GPIO18 HIGH → NO open → switch untouched (default) GPIO18 LOW → NO closes → switch resets (1s pulse) Switch has its own 18V AC power supply. |
| --- | --- |

|  | TUNABLE VALUES — Change These to Adjust Behaviour |
| --- | --- |

| Setting | Default | What to do if it needs adjusting |
| --- | --- | --- |
| AUTO_INNER_INTERVAL_MS | 30000ms (30s) — Increase to 45000 for 45s. Decrease for more frequent inner loops. |
| PARKING_DELAY_MS | 2000ms (2s) — Increase if inner train overshoots parking spot. Decrease if stops too early. |
| RESUME_DELAY_MS | 1500ms — Increase if outer train needs more time to clear zone before resuming. |
| SPEED_RAMP_MS | 1500ms — Time at intermediate speed before returning to user speed. Increase for smoother ramp. |
| RESUME_RAMP_SPEED | 5 / 7 — Intermediate speed during ramp-up. Lower = gentler resume. |
| DEFAULT_OUTER_SPEED | 7 / 7 — Speed outer train runs at on boot, before user changes it. |
| ZONE_TIMEOUT_MS | 30000ms — Force-resume if Sensor C never fires. Safety net for stuck trains. |

|  | SERIAL MONITOR COMMANDS |
| --- | --- |

| Key | What it does |
| --- | --- |
| 1-7 | Set outer train speed. This speed is saved and restored automatically after any collision stop. |
| + / - | Increase or decrease outer train speed by one step. Saved speed updates too. |
| s / e | Stop outer train (Emergency stop). |
| f | Set outer train direction to Forward. |
| r | Set outer train direction to Reverse. |
| X | Manually trigger inner train immediately — runs one loop then parks. Safe even if zone is clear. |
| z | Emergency cut inner train power immediately. |
| p | Manually pulse track switch relay (GPIO18) for 1 second. |
| h | Toggle horn ON / OFF. |
| b | Toggle bell ON / OFF. |
| l | Toggle lights ON / OFF. |
| n / m | Sound ON (max volume) / Sound OFF. |
| v | Cycle volume: 1 → 3 → 5 → 7 → 1. |
| a | Play announcement / conductor speech. |
| i | Show full diagnostics — speed, relay states, sensor states, errors, timers. |
| ? | Show this command menu. |

DATIX AI — Intelligent Systems & Embedded AI Ahmed Ali, CEO ¦ Harry Locomotive Project ¦ Firmware v5.1 ¦ May 2026